Swapping out Oleds on Studer ON AIR 1500 V2  July 2021

Note corrected login and password into

If your OLEDs at the top of each channel go dim, you will need to swap them for new ones.

Do a USB config backup !!!!!!!!!!!!!!!

See software instructions at the end

You need new Oleds, a 3mm allen key, a 2.5mm allen key, fingernails, insulating tape, good light

Studer instructions, 5 new files.

Unplug from the RJ45 cable, so the panel is off !

Remove side, top and bottom aluminium trim using 3mm allen keys.

Put 2 or 3 strips of insulating tape across the oled windows to stop the windows falling out

Sheets of white A4 paper are useful to put the parts on, and or a pot or magnetic saucer

Turn over, protecting faders and knobs, bubble wrap or a cushion.

Pull off the fader knobs, these pull off with your fingernails

Remove fader knobs and and make sure faders are not all the way up

There are 12 allen screws, remove with a 2.5mm allen key. Don’t remove the countersunk screws.

Remove black back panel to expose underside of PCB.

Unclip the plastic clips holding the PCB in.  This is fiddly.  Remove the PCB, taking care not to drop the OLED covers.

Once PCB is out, remove the OLED covers or pick them up off the floor, if you used tape they will remain in place

Remove the whitish rubber moulding, these are reused

Lift up the OLED with a finger nail

Remove the black rubber oled holder, there are 4 rubber spikes on the other side of the PCB, put to one side these are reused

Lift up the brown clip to release the ribbon cable.  Note the gold contacts are down.

Install new OLED, it needs to be folded.  Gold contacts down !

Install the new OLED and clip the brown clip back down.

Replace the black rubber oled holder, pull the spikes tight from the other side of the board.

Press the oled into the black oled holder, they may not sit down too well

Replace the whitish rubber moulding.

Repeat as required.

Clip PCB back into top of module, carefully, align rubber buttons, faders in middle of travel!

The PGM / REC modules and oled windows may not sit right until the screws on the back are tightened.

Put aluminium trim back on, top and bottom have the longer edge at top.  Tighten 12 screws on back gradually.

Check everything is seated correctly.

Reconnect to Core and test, the oled images should be mirror imaged

If you follow the supplied studer instuctions, they are almost correct.

Plug in a dvi-d screen, Mouse click on the Studer splash screen as the system boots to get the IP address.   Save and return x 3.

Or use system viewer to get IP address

Or use remote console if you don’t have a dedicated screen for metering

Or Use your laptop in the same range as the core.

Ping the IP address of the core.

Use Filezilla, using the IP address of the core, not as stated in the instructions.

Use Port 21.

Enter the user name todesursache   and password   pilzvergiftung      
“Cause of death, poison mushrooms” if you were wondering.

Follow instructions and copy a backup of the firmware to your laptop.

Rename to remove the OA1500_  and OA2500_  prefix from file names and copy into the firmware folder as per studer instructions.  You can only rename the files if you have extracted them from the zip file

Make sure surfaces are connected, the oleds should be reversed.

Log onto the Studer GUI as admin, open admin tab, click on firmware tab.

You are hopefully on v6.1 build 4

Scroll down to Desk driver V94.0 ( for V6.1 ), do this by right clicking, holding down the button and scrolling left and right in the black box shown above.

Port 0 is the large surface, Port 8 is the smaller surface

Highlight Port 0, and click on the update firmware box, just visible above.

The LH oled will change and show the files being loaded with a counter, this may take 3 minutes.

Repeat for Port 8.

During the update the screen above will show V0.00, but then change to V0.13 or v0.14 as above, Port 8 should change from V0.10 to V0.11

When update is complete ( note  Klingon Option -     do not switch off power during this )   reboot core.

Reboot the core from the config screen,   Oleds should now display correctly.

Now check desk operation.

Trouble shooting

1/ You can’t see the studer core on FTP :

Use filezilla, it works

check login and passwords, 2 x s in todesursache, lower case p in pilzvergiftung. Port 21

check you can ping it.

2/ Can’t rename the files to remove the prefixes.    Extract files from zip.

You can copy all 5 in at once, takes seconds

3/ You’ve copied the files in and rebooted, but oleds are still mirror image.   You need to update firmware on both surfaces.

A reinstall of V6.1 or an update of V6.0 to V6.1  will not contain the new oled drivers, they are a patch.